E:\PAGESEW\RUFFTEXT\\ROUGH024.TXT

Rough Sewing:  File 24:  Mending

MENDING

Table of Contents:

General principles

Missing Stitches

Missing Fabric

Darning

darning hand knits

mending factory knits

darning woven fabrics

Patching &&

Replacing Fasteners

Bed Linen

Mending Woven Wool

---------------------------------------------------

codes:

&& marks incomplete or missing discussion

ignore whole line:  it's a printing code

..comment you aren't supposed to see

L---P----1----+----2----+----3----+@10-4----T----5----R----+----r----7--T-+--r

General principles

==================

Whole books have been written about mending, so

this chapter will be rather sketchy no matter how long

it turns out to be.

Though I can give you specific instructions for

making things, only examples and general instructions

can be given for repairing things -- you have to deal

with what you've got, so ingenuity is always in demand.

Mending can be divided roughly into replacing

missing stitches and replacing missing fabric.

Roughly.  Sometimes the stitches are missing

because the fabric has worn away.

Missing Stitches

================

The easiest damage to repair is missing stitches.

Sometimes you can re-sew a popped seam the same way it

was sewn in the first place.  More often, you will have

to replace machine stitches with hand stitches, because

you can't get at the spot with a machine, or because

you need to re-use the original holes.

Usually, you will replace machine stitching with

back stitches -- the strain that broke the original

stitches is probably too much for running stitch, and

closed back stitch looks like machine stitching on one

side.

Sometimes you can't get at the inside of the seam,

and must slip-stitch it from the outside.

Should you need to simulate the appearance of

machine stitching on both sides, use double running

stitch.

When possible, thread the ends of the broken

threads into a needle and weave them back into the

remaining stitches, or slip them between layers, or

hide them inside a thick fabric.  Or continue the back-

stitch for half an inch or so to each side of the gap,

securing the machine stitches by overlapping.

When machine-stitching a gap, take the first stitch

just beside the line of stitching so that the ends

won't pop through to the right side, then stitch

exactly on the old stitches almost to their end, stitch

on the other side of the old stitches so that *their*

ends won't pop through, and reverse the procedure on

the other side of the gap.

Missing Fabric

==============

When fabric is missing, you can darn it, sew on a patch,

glue on a patch, or darn on a patch.  Or cut the garment

shorter, or remove the damaged fabric and replace it.

When you glue on a patch, it is wise to sew around the

edges to prevent them from working loose.  A patch with

loose edges will eventually peel off, but the glue will hold

if peeling doesn't have a place to start.

Darning

=======

Machine Darning

---------------

When appearance is not important, you can darn by

machine.  If appearance is *really* unimportant, you

can use up the left-overs and mistakes in your thread

collection -- thread too weak for sewing may be plenty

strong for darning.

Darning by machine requires a steady hand.  As

compensation, if you do a lot of it, you'll be able to

do free-motion embroidery.

A special darning foot helps a great deal, but you

can darn with no foot at all in the machine.  There is

also a special needle for free-motion embroidery, with

a spring to hold the fabric down.

In any case, set the stitch length for zero and, if

possible, lower or cover the feed dogs.  You must lower

the presser foot bar even if there is no presser foot

on it -- the machine releases the thread tension when

the foot is up.

Starching the fabric helps, and stretching it in an

embroidery hoop is essential.  There are special thin

hoops that can slip under the presser foot, but any

hand-held hoop can be used.  When using an ordinary

hoop, you will probably have to slip it under with the

darning foot off, and may be obliged to remove the

needle.  Make sure the fabric is on the bottom when the

side you want to stitch on is up!  (When the hoop is

used for hand embroidery, it's the other way around.)

I have seen darns made by swirling around

chaotically, but you get a flatter darn if you make two

sets of parallel lines, intersecting at right angles.

Make each line extend well into the sound fabric, a

different distance for each line, so that the ends

don't line up to make a stress riser.  You need only

one or two lines on each side of the weak spot, so the

darn will approximate a Red Cross emblem in shape.

Sharp turns are to be avoided as much as you can:

a sharp corner is a good place to start a tear.  If you

make the lines of stitching twice as far apart as you

want them, then fill in with a second pass, it will be

easier to make smooth turns.

You can cover a hole with a grid of stitching, then

make a second, smaller darn to thicken the thin spot.

A better plan is to glue a patch of thin fabric into

the hole before darning.  Iron-on fabric will do, since

the primary strength of the darn comes from the lines

of stitching.  Starch, glue-sticks, and various iron-on

tissues and webs can also baste a patch in place.  Even

when the glue is supposed to be permanent, presume that

it is going to wash out, and stitch accordingly.

Patch-reinforced darns merge gradually into patches

darned or quilted onto the fabric. Lines of stitching

that darn on a patch will be farther apart than

stitching that is intended to replace fabric.  You can

zig-zag over the raw edges after darning, but this is

seldom necessary.

Darned-in patches cover both extremes:  I've

described the quick-and-dirty minimum-effort patch

above, and in the section on mending woven wools, I'll

describe the invisible-mending patch that is darned in

by hand.

Hand Darning

------------

Mending hand knits

------------------

<!--

.. discard line:    Whether handmade or not, a worn synthetic mitten should probably be discarded --- when one part has worn thin, the rest is pilled and shabby, and synthetics are hard to darn.

..caption for diagram of buttonhole stitch:  A series of simple half hitches is peculiarly suited for working over an edge, so it is much used in making buttonholes, and sometimes was used to secure the edges of fabrics too heavy to hem -- such as blankets

-->

Darning wool is no longer available, so you will have to

use embroidery wool.

Persian wool is good for darning worsted-weight knits,

and for replacing fingering-weight that's missing

altogether.

DMC makes a very fine embroidery wool they call

"Medici;" it's a bit hard and tight, but it's the only

easily-available wool fine enough to darn factory-knit

fabrics.  Two or more strands of Medici make a flatter darn

than a single strand of a heavier yarn.  (Medici is now

available only at thrift shops and "we cleaned the

warehouse" sales.  Use the finest crewel wool you can

find.)

Some yarns spun for warp are strong enough to darn with;

if you know a weaver, ask him to save selected thrums -- the

waste bits of yarn that are left when fabric is cut off a

loom are plenty long enough for darning and embroidery.

Since darning uses up thread faster than sewing or

embroidery, you may need to use longer pieces of yarn than

you would ordinarily.  When using an over-long thread, keep

the yarn and its tail about the same length, to minimize

tiresome reaching.  This also forces you to shift the needle

every few stitches, which reduces the tendency for the yarn

to wear through where it's folded.  If the yarn tends to

twist and snarl, you can combine needle-shifting with

dangling:  drop the needle, and wait until it stops

spinning.  Then unfold the tail, drop it straight, and wait

until that stops spinning.  Then push the needle down half

an inch -- more if you are taking very large stitches -- and

resume work.

When sewing with silk, push the needle up to the

fabric and hold it there while letting the thread

dangle.

Long "darning needles" are for mending woven fabric, and

are not suitable for darning knitwear.  Use a short, blunt-

pointed needle with a long eye, just large enough to make a

hole that your doubled mending yarn can pass through easily.

Canvas embroidery is the most common use for needles of this

type, so they are called "Tapestry needles."   Look for them

in most of the places that sell hand-sewing needles.

Sharp-pointed needles of the same type are sometimes

useful, particularly when the yarn isn't wool, but your

usual goal will be to encase the worn strands without

piercing them.  If you darn with plant fiber or synthetic,

it's a good idea to switch to a sharp-pointed needle for

securing the ends, but wool will hold when merely slipped

under the network of darning.

Work proceeds much more easily if the fabric is

stretched over a hard, smooth, curved surface.  An object

put under a worn spot to provide this surface is called a

"darning egg", because the most common type is an egg-shaped

piece of wood mounted on a handle.  The handle doubles as a

darning surface for glove fingers.  The handles of knives

and kitchen tools have also been used as darning eggs in

glove fingers.

Rock shops sell egg-shaped stones in assorted sizes, and

craft shops sell wooden and plastic eggs.  At Easter time,

you can buy hollow eggs that double as a place to store your

needle and yarn.  The kind with one transparent end are the

most convenient.

Real eggs have been pressed into service -- it's

considered wise to boil them first, in case a cat catches

its claw in the work, panics, and dashes around smashing the

egg against walls and furniture.

It was a hundred-watt light bulb that met this fate.

Light bulbs are just the right size and shape to darn a

stocking heel, and are always handy, but if the cat is

around, light bulbs in my socks make me nervous.

My grandmother used a small gourd that has a finger-

sized neck and a bulbous bottom.  I find my nest of

cylinder-shaped stainless-steel mixing bowls handy when

darning bicycle tights.  Look around.

After much experiment, I have concluded that the best

way to darn knitting is to work interlocking rows of

buttonhole stitch, also called "blanket stitch".  (Mildred

Graves Ryan calls this "point de venise darning".)

<[photograph at <br><a href="../PVENISED.HTM">Point de

Venise</a>:&nbsp; overlapping rows of buttonhole stitch on

the heel of a sock.]>

A buttonhole darn is elastic, it covers the

weakened fibers on both sides, and it can be tapered by

working larger stitches on the sounder parts of the

fabric.  If a mitten keeps wearing through at the same

spot, one darn can be worked over another without

making lumps.  When filling holes, it can be worked

over horizontal strands thrown across the hole, and

will cover any vertical or odd-angled strands you have

used to stabilize the shape of the hole.  You don't

have to cut away odd shreds, but can buttonhole them

into the darn.  This helps in "feathering" the edges of

the hole.

Rows of buttonhole should be straight, even when

the hole is round -- curved rows pucker.  Usually, I

work parallel to the cross grain, but buttonholing also

works on the vertical grain.  Vertical and horizontal

patches of buttonhole darning co-exist peacefully.

Sometimes it's a good idea to work a darn that just

fills in the hole, making the stitches loose enough to

match the thin fabric around the hole, and then work a

reinforcing darn over the entire area.  Or one can work

a very loose darn as permanent basting to hold a large

hole in shape while serious darning is done.

I have been known to work a loose woven darn as

scaffolding -- weave a thin yarn up every column of

stitches, being particularly careful to catch the raw

loops at the edge of the hole.  Begin in an intact

column so that you'll be able to see where to resume

weaving after crossing  the hole.  Since weaving up a

column doesn't show on the right side, you can continue

well into the sound fabric to reinforce it against

further wear.  Properly, you should break the yarn

after every row to minimize the restriction of stretch,

but you can get away with leaving a small loop where

you turn.  Then weave yarns into the rows, weaving over

and under the vertical yarns.  I suggest weaving in the

rows only for stocking heels, which tend to have all

the stretch felted out of them anyway.

If the mitten or stocking is light in color, a water-

erasable marker (sold in sewing, embroidery, and quilting

shops) can help you to keep your stitching straight.  Before

starting to darn, mark carefully along rows of stitches

spaced about half an inch apart -- or more, or less,

according to the situation.

If you are having a lot of trouble, or want to be

particularly fussy, you can weave a smooth sewing thread

through a row of stitches to serve as a guide.  Since you

are working with a blunt needle that doesn't pierce threads,

the contrast thread can be pulled out after you work over

it.  Or use a silk thread and leave it in.

Since these threads will be covered by the darning

stitches, I use whatever silk thread I want to get rid of

that isn't a startling contrast to the darning yarn.  This

is one time that I think spun-silk thread would be better

than reeled-silk thread, but I won't buy a spool of spun

silk to try it.  (Perhaps I will when I've used up all my

old thread of questionable strength.)

At first, I wove the silk over and under whole stitches,

as I do when marking every tenth round in my knitting, but I

soon realized that it was easier and plainer to go under one

leg and over two.  That is (being right-handed and therefore

working from right to left), I would go under the left leg

of a stitch, over the next stitch, under the right leg of

the stitch after that, then over the left leg of that stitch

and the right leg of the next stitch to pick up a left leg.

When confronted with a mesh of stitches so thin and

wispy that I was afraid to utter a harsh word in their

presence, I ran a silk guide thread through every stitch of

every row, going under the right legs because it seemed

easier to come up in the stitch.

Be sure the work is well stretched over the darning egg

while marking, so that the ends of the threads won't pull in

when you stretch the fabric to darn.  (If you plan to pull

out the thread, you can leave short tails instead.)

Pre-stretching also makes the silk threads somewhat wavy

when the tension is released, which will helps to keep them

from restricting the stretch of the buttonholing.  (You are

primarily counting on their slickness, which prevents them

from grabbing the wool threads to hold them back.  By the

time they have been worn and washed enough to felt in, they

will be serpentine.)

After I used up my buttonhole twist of doubtful

strength, I selected size A sewing silk of a nondescript

gray, thinking that this color would disappear into the

darn.  It slipped out of the needle and disappeared

prematurely.  After I'd given up searching for the piece

I'd cut off, I threaded the needle without cutting off from

the spool, wove it through, then pulled it back and cut it

off where I'd begun the stitching.  No more losing the

thread, and no more running out.

Where the fabric is thinner, you'll need to make smaller

stitches.  This means that you'll usually need more stitches

and rows in the middle than you need around the edges.

This, in turn, means that you'll have to increase, decrease,

and make short rows even if you never darn a curved surface.

To increase, work two stitches in one stitch.  If this

seems inclined to pull a hole, make the second stitch

longer, and catch it in the previous row.  Or you can make

one stitch over the thread only, and catch the fabric in the

other stitch.

To decrease, skip over a stitch.  Try to keep the

stitches uniform.  If a narrower-than-average stitch in the

previous row presents itself, skip that one.  Stitch as

close as possible to the skipped stitch when working into

the stitches before and after it, so that the stitch which

spans the skipped stitch is as narrow as possible.  This

stitch will be wider than average anyway, but the extra

width will be divided between two stitches when you work the

next row.  (Since every stitch spans half of each of two

stitches in the previous row, irregularities tend to average

themselves out.)

To turn a short row, make the last stitch less tall than

the others, then put the needle down where you would if you

were making another stitch, and bring it up where you would

have put it down in the next stitch.  This causes the top

line of the row to dive down into the top line of the

previous row.  Another plan is to slip the yarn under the

top line of the stitch you would have stitched in, then slip

it under the bar between that stitch and the next, and out

under the top line.

If you don't want to begin the next short row from where

you are, slide the thread under the stitches, as if hiding

an end, and end by going down in one stitch and coming up in

the next.  (Or by exiting under the top line.)

Try to make your first exercise in darning the covering

of a weakness that you have caught so early that you can

work a uniform net, thin enough not to have a definite edge,

over the entire thin spot and a bit of the sound fabric

around it.

To cover a thinning spot in buttonhole:

Begin by weaving the yarn up a stockinet column the same

way you hide an end when knitting:  down in one stitch and

up in the next.  Subsequent beginnings should be secured by

sliding them under a row of buttonhole stitches, between the

darn and the fabric.  If you want to get an end out of your

way immediately, instead of weaving it into the darn as

suggested below, weave it over and under stitches like the

first beginning.  This disposal is particularly suitable

when you have worked the end down too short to thread

into a needle:  weave the needle, *then* thread it.

If a short end gives you fits, use a crochet hook to

pull it in.  It's usually easier to use the needle, but

"usually" isn't "always".  You can also use a fine thread,

as suggested in "Hiding Short Ends" at the beginning of

"Hand-Sewing Stitches"

If the darn is stretched parallel to a hidden end, the

end will pull back into waves and be less inclined to

restrict the stretch of the fabric.  It is rare for an end

to get felted into place before it gets stretched, so you

don't have to worry much about ends that are slid in after

the stitches are made.  When stitches are worked over a

yarn, the yarn is less inclined to slip, and when both ends

of the yarn are secured before stitches are worked over it,

as sometimes happens when padding threads are thrown across

a hole, you definitely restrict the stretch of the fabric.

Whether working over a yarn is good, bad, or indifferent

depends on the stretch of the yarn, the stretch of the piece

being repaired, and the size of the darn.

Try to put your ends where they will do some good.

You usually should work on the right side.  The side

next to the darning egg is smoother and more suitable for

wearing next to the skin.  The side that is uppermost when

you are darning has more yarn in it, and is suitable for

taking wear.  If the wear came from the inside of the

garment -- perhaps from a ring or an orthopedic device --

turn it inside out, and darn on the side where the wear is.

I'm going to describe the darn as if the rows were

horizontal and the fabric vertical.  Turn it to the most-

convenient angle.  Right-handers will probably want "up" to

slant away and to the left.

Come up below the worn spot and a little to its right,

and buttonhole over a row of stitches, right to left.  Use

the knitting as a guide to make your embroidery stitches

uniform and square, about as high as they are wide.  (Making

them narrower without making them shorter is one way to

thicken the middle of a row, but square is the best shape

for working a net over an area.)

When you are a little to the left of the thin spot, end

the row by putting the needle down where you would have

brought it up in making the next stitch.  That is, the last

loop is secured by a short straight stitch that ends where

the next loop would have had its corner.

Bring the needle up in the right place to begin the next

row.  This stitch is about half as long as the stitches you

have been making, so it's easy to begin the next row too

high.  If the previous row was a trifle too long, begin by

working into the last loop.  If the previous row was a

trifle too short, work into the straight stitch securing the

last loop as if it were the top of a stitch

Use the columns of knit stitches as a guide for making

the side edges straight, unless you have a reason to make

them some other shape.  Try to keep the underneath stitches

vertical.

Work back left to right, turn in the same way, and

continue until your thread gets short.

When changing threads in ornamental buttonhole, one

leaves the last stitch unfinished, and later weaves the end

through the first stitch of the new thread to give the

illusion of an unbroken thread.  This is both inconvenient

and undesirable in darning, where the ends are woven in on

the right side.  Temporarily secure the old end by taking a

stitch well above the darn, unless you have worked the end

down so short that leaving it flopping doesn't bother you.

Slide the new thread under the row below the last row of

darning, then take a stitch that comes up in the same place

where the old end comes up, and continue darning.  When you

have worked a row, thread the old end into a needle and

slide it under the second row from the edge, which is now

the row above the row where you secured the new thread.  If

you need to work past the old end, slip the needle under it

to avoid nailing it down prematurely.

When wear begins abruptly, as sometimes happens in

striped stockings when one yarn is more durable than

another, make the row of stitches in the sound fabric twice

as large as you will need in the thin spot, then work

two stitches into every stitch on the second row.

Though working over a thread reduces the stretch of a

darn, a hole fills in so much faster when the stitches are

padded that it is often worth it.  You can throw a short

yarn across the hole before each row, weaving the ends well

to both sides of the hole, you can take advantage of ends

that need hiding, or you can stitch across in one direction

and strand back.  This last appeals particularly to people

who buttonhole more easily in one direction than in the

other.

Another approach is to first stabilize the shape of the

hole with a loose network or a zig-zag of yarns, and work

over them as you come to them.

All these methods can be combined, of course.

Other darns and patches for hand knits:

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

An inconspicuous darn is to embroider rows of chain

stitch up the columns of stitches.  This does not reinforce

worn fabric, but will protect new fabric from abrasion -- in

the spot where a rough gear-shift lever hits the thumb of a

mitten, for example.

You can cover or replace a strand of yarn by duplicate

stitch or grafting.  Since this is tedious work, it's

usually done only when the damaged area is small and the

garment is particularly valuable.

Duplicate stitch is a good "save" when you notice a

defect in knitting yarn after completing the work.  Break a

short piece of yarn by pulling fibers, as for splicing, and

put the middle over the weak spot, then darn away from it in

both directions.  (Use a crochet hook when the ends get too

short for a needle.)  The tapered tails will blend

imperceptably into the original yarn.  If the yarn is

untreated wool, this repair will become more firmly attached

and harder to see with wear and washing.

You can duplicate stitch up to a hole and work

nalbinding across it, if you have allowed something that is

worth that much effort to wear into holes.  A silk thread or

fine yarn woven into each column of stitches will make the

nalbinding easier.

Some people knit patches, by picking up live stitches at

the bottom of the hole and grafting at the top.  The sides

can be sewn, or secured by pulling the frayed yarns through

the stitches.  (The turning yarn that spans from one row to

the next is a good place to link.)  Weave the frayed ends

into the old fabric, to reinforce it.

It may be a good idea to wrap the turning yarns around a

smooth thread, to make it easier to find the place to link

the worn yarns through.

I've never encountered a hole for which a knitted patch

was suitable, so this description is not from experience.

When the end of a finger, the toe of a sock, the cuff of

a sleeve, or some other extremity becomes worn, it's often

suitable to ravel it out and re-knit it.  Sometimes the

original yarn can be re-used for the repair.  Wash it first,

to relax the kinks.  Overlap the frayed ends, instead of

cutting them off and joining as you would join new yarn.

Make sure you stretch each piece of yarn firmly, to see

whether there is a weak spot in it.

If the extremity was knitted the other way, and won't

unravel from its beginning, snip one thread at the spot

where you want to divide the fabric, and pick it out all the

way around.  The piece that falls off usually can be

unraveled from this side, if you need to incorporate the

original yarn into the repair.

Mending factory knits

---------------------

If worth the effort, a factory knit can be darned like

hand knits.  If you have matching fabric -- preferably from

a worse-worn copy of the same garment -- it can be patched.

Hand-sew the patch with Medici or spun silk, and overcast

the edges down instead of turning them under.

(Above remarks are for wool; I discard synthetic factory

knits when they are badly worn, so I have no experience to

share.)

Baseball-stitch darning

~~~~~~~~~~~~~~~~~~~~~~~

A frequent failure in cut-and-sewn garments is fabric

that has worn away along a seam.  Seams tend to wear through

at the fold, and a sharp, hard line of overlock stitches may

serve as an anvil on which the fabric can be worn away.

One way to repair such a seam is to re-stitch the seam

with baseball stitch, spaced closely enough to overcast both

raw edges.  Since this seam is very flat, it has little

tendency to wear away.

(In some references, baseball stitch is called "antique

seam" because it was used for construction in the days when

fabric was so valuable that allowing it to wear away at the

seams was unthinkable, and so narrow that you were apt to

have seams in places where lumps and ridges are

intolerable.)

First cut away the seam allowances, not only where they

have worn free of the main fabric, but into the sound fabric

on either side.  (Baseball stitch is easy to work next to a

previous patch of baseball stitch, so you needn't be

fanatical about preventing future failure.)

Choose a thread that harmonizes with your fabric.

Secure the thread very thoroughly if it is synthetic, merely

hide the end if it is wool, take intermediate precautions

with silk and cotton.  Sliding the thread through a serged

seam is a good way to hide an end; taking back stitches in

the overcasting is a good way to secure an end.

Begin stitching a little above the break, stitching

toward the seam from both sides so that the stitches

interdigitate across the seam.

..PICTURE NEEDED HERE

When you get to the break, let each stitch come up in

the gap, so that the edges are overcast and held together by

a figure eight, and the crossing of the threads in the gap

prevents the edges from over-riding one another.  Stitch

beyond the end the same way you stitched before the

beginning, and hide or secure the end.

When the gap is too wide for baseball stitch, you can

fill it with one or more vertical threads and darn over

them.  If the number of threads is odd, the yarn will come

up through the fabric on one side and go down into the

fabric on the other.

Darning Wovens

--------------

Sometimes the poInt de venise darn described for hand

knits is appropriate for woven fabric.  More often, you will

use a woven darn or a running-stitch darn.

You make a running-stitch darn by running back and forth

across the thin spot, stitching parallel to the threads of

the fabric.  Leave a very small loop of thread at each place

where you turn to go back, and try not to have all the turns

pulling on the same thread of the fabric.

Sometimes only one set of threads is weak, but usually

you will need a second set at right angles to the first,

making a cross-shaped darn like the machine darn described

above.

If there is a hole in the fabric, darn as for thin

fabric, making the rows of running stitch that cross the

hole closer together than those that are entirely in fabric.

Then lay a second set of running-stitch rows at right angles

to the first one, weaving over and under the strands that

the first set left across the hole.

Most darns are plain tabby weave:  over one thread,

under the next, and always go over the threads that the

previous row went under.  But it is possible, if the fabric

is coarse or you have very good eyes, to duplicate various

fancy weaves.  In this case, you carefully follow the course

of the threads through the fabric, instead of leaving

conspicuous running stitches on all sides of the darn.

If it is difficult to fill up the hole without unduly

crowding the stitches in the thin fabric, space the first

set of threads as for thin fabric all the way across, then

when making the second set of stitches, run the thread up to

the edge of the hole, then chain stitch across it, making

one stitch over each thread all the way across the hole.

You can, of course, begin chain stitching a little before

you actually reach the hole.  Chain-stitch darning also

serves when the threads are fuzzy, or for some other reason

you can't see to weave over and under.

&&

Patches

=======

When fabric is worn thin, it may be covered or

replaced with a patch.  There are tips on patching

wools and knits scattered through the section on

darning.  Here, I will discuss patching in general,

then describe a pieced patch for the crotch of pants.

After determining that the garment is worth

patching, you must ask yourself whether you have

matching fabric, and whether you can tolerate

contrasting fabric.

The best patches are usually taken from a worn-out

garment of the same fabric, second-best, scraps from

making the garment to be mended.  New fabric may be

best if you are repairing a spot that takes hard wear

and the fabric around it is still quite sound.  New

fabric is contra-indicated when the fabric near the

spot to be mended has worn thin and soft; since a

garment in this state has usually reached slopping-

around status, a contrasting patch of a thinner fabric

is acceptable, and it will cover the damage without

stressing the worn fabric.

EXCERPTS FROM POSTS TO VARIOIUS FORA

stashed here in case bits of them are usable

-------------------

I'd probably steal a patch from the hem to cover

the hole, and sew it on by hand with spaced back

stitches and matching thread.  On the side that shows,

make very short stitches parallel to the threads of the

patch, so that they can sink down between them.

A patch should be significantly larger than the

worn area.

Open the hem before sewing on the patch.

If you lack confidence, baste the turn-under of the

patch before pressing it, then baste the patch onto the

dress with thread of a different color and pull out the

first basting.  Pressing after basting the patch seems

to make it more willing to stay put while you sew.

If the style of the dress permits, consider

covering the damage with

rick-rack, ribbon, etc.

-------------------

The patching section of the chapter on mending is

expanding like a bowl of yeast -- before I can explain

anything, I have to explain something else first.  With

the weather having turned suitable for gardening and

long bike rides, getting it written is going to take a

while.

And what you want to know is much more limited:  I

suggested that you cover the damage with a piece of

matching fabric -- if you happen to have some scraps

from making the skirt, use those; if the hem is wide

enough, you can cut a patch out of the part that

doesn't show, then repair the hem with whatever fabric

you have.

The patch must be large enough that all stitches

are taken in sound fabric.

Being denim, the patch will need to have its edges

turned to the wrong side to keep it from ravelling.

I'd suggest cutting the patch square to make turning

the edges under easier.  Cut exactly along threads if

you can, as on-grain edges are easier to handle.  Don't

forget to allow for both the turn-under and a seam-

allowance clearance around the hole.

Turn the corners first -- if basting the creases

in, fold each corner over your needle to make it keep a

straight line.  If pressing, use a thin metal ruler or

a business card.  (For such a small patch, basting is

easier.)  Then fold all the edges to the wrong side.

You needn't meet in a neat miter at the corners --

there is something to be said for blunt corners on a

patch -- so you can eyeball, rather than measure, all

distances.  Denim might hold a crease when pinched; if

not, baste the corner creases in, then pull out those

short bits of thread after basting the edge creases in.

After basting the creases, press the patch.

Everything behaves better after being swatted with an

iron.

Lay the patch over the damage and line up the grain

of the patch with the grain of the fabric.  Stick a pin

in to hold it in that position while you baste close

to, but not through, the basting that holds the

creases.  Also avoid the line where you intend to place

the final stitching.  Remove the basting that holds the

crease, as the basting that holds the patch in place

will now do the job.  (Hence my suggestion of using two

colors of basting thread.)

Now spaced-backstitch around, making the stitches

as small as you can manage (but not *smaller*; each

stitch must catch at least two threads of the fabric):

Slide the needle under the patch to hide the end of

the thread; securing the end is not necessary, as

backstitch is in itself an end-securer. Come up close

to the folded edge of the patch.  If right handed,

stick the needle in about a sixteenth of an inch to the

right of the place you came up, trying to make this

hole between the same two threads as the hole where you

came up.  Bring the needle point up about an eighth of

an inch beyond the place where you came up the first

time.  If you can't firmly catch the bottom layer with

a stitch this short, make it a little longer, or make

the stitch in two steps.

Theoretically, the stitches on the back will be

three times the length of the stitches on the front;

ideally, they will be the same length as the stitches

on the front because of slanting through the thickness

of the fabric.  Practically:  if the thread catches

enough fabric that it isn't going to tear loose, be

happy.

When you get back to where you started, slide the

thread under the patch again, and snip it close to the

fabric while it is under tension, so that the end pulls

back inside.

Remember that I intend to incorporate this

explanation into a work in progress; if you ask for

more information, or find fault with my writing, you

are doing me a favor.

(A rare favor; you'd be amazed at how much work

picking nits is, and how few people are willing to do

it.)

-------------------

&& && &&

One way to patch the crotch in pants

------------------------------------

Pants sometimes wear through at the tops of the

inseams while there is still enough wear left in the

rest of the garment to make it worth your while to mend

them.  If you have the original pattern and some

matching fabric, you can replace the worn area with a

gusset.

The best patch fabric is the good parts of a worn-

out garment made from the same fabric; next is scraps

left from making the pants.

Wool scraps will probably match pretty well.

Cotton, particularly blue denim, is likely to have

faded enough to make the patch an embarrassing

contrast.

If the fabric is 100% plant fiber, you can zig-zag

or overlock the edges of a scrap large enough to make

your patch and wash it with your dish towels (or

anything else that you bleach), and dry it in the sun,

until it's a bit less new.

It's best to begin this process before patching

becomes urgent, as it may take a while.  Artificial

fading always reduces the life of fabric, so don't do

it unless you need to.  Remember that bleach dissolves

animal fibers, and sunlight turns wool yellow.

Having obtained matching fabric, make a gusset

pattern.

Trace off the crotch corners of the pants back and

pants front onto one paper, matching the inseam

stitching lines of the original patterns to make a

single new pattern.

Measure the worn area and mark a larger area on

your pattern, then add a turn-under.  Let the outline

of the patch cross the crotch seam line at an angle

between forty-five degrees and a right angle* at both

ends, and curve smoothly everywhere else.  Strive for a

shape that might have been sewn into the pants in the

first place.  For example, if you got the patch fabric

by converting long pants into shorts, let the gusset

extend all the way to the new hem line.  (In this case,

the seam line of the patch will cross the new hem line

at right angles.  It might be a good idea to make the

patch half an inch too long and trim it after it's

installed.)

<footnote> * The patch should have a slight obtuse

point at its seam to correct an illusion that it is

indented there. </footnote>

Cut two pieces, one right and one left, with the

inseam line on the straight of grain.  Mark them so

that you don't forget which side is front and which is

back.  Also mark the old inseam line, which is now a

grain line, so that you can match it when installing

the patch.

Sew the pieces together in a mock-fell seam,

pressing it the opposite way to the crotch seam in the

pants.  If the front-center and back-center seams point

opposite ways, decide which end of the seam will be

more conspicuous and de-lump that one.  Probably the

back, since you sit on that part of the seam.

Press under the raw edge and appliqu the patch to

the pants, top stitching twice to match the mock-fell

seams in the pants.

Now cut away the worn fabric under the patch.  This

is very important -- if you leave the inseams under the

gusset,

they will wear away the new fabric.

You can match felled seams by cutting away after

edge-stitching around the patch, then turning the cut

edge of the pants under and top-stitching again, but

this is rather lumpy, and no-one who doesn't see the

inside of the pants will know that you have two kinds

of seam.  On the third hand, you might have had a

reason for flat-felling the seams, and it might still

apply.

If the pants seams are neither flat felled nor

mock-felled, ingenuity is required.  You may have to

sew the patch on by hand.

If the rest of the pants are in *perfect* shape, the

fabric may be unusually vulnerable to abrasion, and it won't

wear any better in the form of a patch than it did the first

time.

The patch may last longer if you underline it with a

thin, tough fabric of a compatible fiber.  It isn't

necessary to match the color, but a sharp contrast can

annoy you when you are putting on the pants.

Cut away the turn-under allowance of the underlining,

and fold the patch around the edge of the underlining.

Also cut away the seam allowance of the underlining on the

side that will be concealed by the mock-fell seam.

To piece an underlined patch with a pre-graded

flat-fell seam:

(Note:  I did this once, but didn't take notes.

These instructions may be revised when I do it again.)

Place an underlining piece on a patch piece with

the turn-under sticking out evenly all around except at

the seam.  You may have made a pattern for the

underlining, or you may have trimmed off the turn-under

allowance after cutting it from the patch pattern.  If

your hand is unsteady, you may want to mark the

trimming line after placing the underlining on the

patch.  Don't sweat minor wobbles, as long as the raw

edge of the underlining will be completely covered by

the turn-under, and the underlining will be firmly

caught in the stitching that attaches the patch.

<hr> <!-- ----------------------------------------- -->

Another way:  Cut the underlining and mark it, then

use it as a pattern to cut the fashion fabric.  Cut a

quarter inch outside the lining to make a turn-under

allowance.  This will make the patch bigger, but it

probably won't matter.  If it does matter, make a

pattern for the underlining

<hr> <!-- ----------------------------------------- -->

Baste just inside the seam line.  (Or exactly on it

if using a pred-graded flat fell.) Baste elsewhere as

required to keep the lining and patch in registration.

A line of basting exactly on the grain line will

transfer it to the fashion-fabric side.

The lining will tend to cling to the patch if you

press after basting -- if you spray a little starch in

between, it may be firmly glued into place.

<hr> <!-- ----------------------------------------- -->

If using the lining as a pattern, press and baste

it before cutting.  Do the seam-line basting after

cutting, so that you can do it by machine.

<hr> <!-- ----------------------------------------- -->

Repeat for the other side of the patch.

Trim a narrow strip from the seam allowance of one

piece of the patch.  Wrap the underlining around the

edge of the patch and press or baste it into place.  (I

find basting easier.)

On the other piece, trim off the seam allowance of

the underlining near the basting.

<hr> <!-- ----------------------------------------- -->

{Deprecated comment which may be needed elsewhere}

On a later patch, I found that one can sew the seam

first, then run a seam ripper between the basting and

the stitching, and peel off the seam allowance.  One

may need to tweeze some threads caught in the

stitching.  Put the *blunt* tine of the seam ripper

between the fabrics.

<hr> <!-- ----------------------------------------- -->

Place the two pieces right sides together with the

raw edge of the patch matching the raw edge of the

wrapped-around underlining.  Stitch the width of your

original seam allowance from the folded edge -- the

exact width trimmed from the one allowance will be

gained back by making the other allowance narrower, and

the original stitching lines will end up superimposed.

Trim excess fabric from the ends of the seam

allowances to make turning the edges under easier.

With the right side up, stitch close to the seam,

keeping the seam allowances underneath flat as you

stitch them into place.  (You can feel them through the

patch as you smooth the work against the bed of the

machine.)  For once, pre-pressing is not required.

Turn the work over and begin stitching at the end

where you left off.  This means that you don't have to

cut the threads, and, more important, means that the

fabric that was pulled by the feed dogs in one

direction is now being pushed by the presser foot in

the same direction, so that you don't get wrinkles in

the seam.

Turn under the edge and applique as directed for

the mock-fell seam.

----------------------------------------------------

Replacing Fasteners

===================

Missing Buttons

---------------

If you are lucky enough to find the button that has

fallen off, take firm measures to keep it from getting lost:

if you can't sew it back on at once, fasten it to the

garment with a safety pin, tie it to the buttonhole with a

piece of string, tie it into the corner of a handkerchief

and put the handkerchief into a zippered pocket of the

garment to which the button belongs.  Nailing a button to

your pincushion with a T-pin will keep it from getting lost,

but it doesn't always get it back together with the garment

it fell from.

If the button is gone for good, you might be able to

find one that's close enough in your button jar.  If the

garment is quite new, there's an off chance that you can go

back where you bought the buttons and buy another, but

nothing in fashion hangs around long, even when it's plain

and ordinary and standard.  If you use lots of similar

buttons -- if you plan to make several shirts with plain

white shirt buttons, for example -- it's wise to buy buttons

by the gross so that buttons that look the same are

interchangeable.

If you can't match the missing button, you may have no

alternative to replacing all of the buttons on the garment,

but it may be possible to steal one from a spot where it's

logical to have a different button or no button at all.

Expensive garments often have a button sewn to an

inconspicuous spot for just this purpose.

Matching the thread is usually easy, but if the thread

was some unobtainable color that calls attention to itself,

you may be stuck with re-sewing all the buttons even if you

have the original button to replace.

Notice how the buttons are sewn on, and sew the replaced

button to match the others.

Hand-sewn buttons are a bit more secure than machine-

sewn buttons, and it isn't likely to be worth your while to

set up a machine for a single button.  It's even less likely

that a button will fall off when you are at home and have a

machine handy.

The technique for attaching a replacement button is

exactly the same as the technique for hand-sewing a button

on a new garment, except that you have to pick out the

broken threads first.

You want to sew the replacement button exactly where the

old button was, so don't pick out the old threads until you

have all the equipment in hand and can get the button sewn

on before the holes close up.  Clipping the broken threads

close to the fabric and pulling on them from the back

usually picks them out cleanly.  Tweezers may help.

It may be necessary to patch or reinforce the fabric

before replacing the button.  If so, put the patch on the

wrong side, and try to keep the stitches in the area that

will be covered when the garment is buttoned.  Since buttons

are sewn to multiple layers, it may be possible to keep the

stitches from coming through to the right side.

Overcast or baseball-stitch torn edges together; with

any luck, most of the darn will be hidden under the button.

(A darn must be backed up with a patch.)

If you are afraid the patch will also be torn, sew a

small, flat button to the inside with the same stitches that

hold on the working button; this will spread out the strain

if the button is jerked.

Think ahead when making clothing, and re-inforce

vulnerable buttons with backing buttons or an extra layer of

fabric.  Bear in mind that when you pull a button, the

threads attaching it pull on the fabric from the back, so it

is on the back that that the fabric needs protection.

Replacing snaps and hooks

-------------------------

Pretty much the same considerations as replacing

buttons, except that it's easier to find a matching hook or

snap, and mis-matched hooks and snaps don't show as much as

mis-matched buttons.  If you have lost only half a snap,

make sure the replacement fits the remaining half, or

replace both halves.

If you find yourself replacing a thread-eye repeatedly,

check the hook for rough spots.  If the hook is wearing the

eye because it's been squashed and is too tight, forcing a

thin knife blade into it may repair it.

It is also possible that, in your zeal for durability,

you are putting too many threads into your bar tack, so that

it is so thick that it has to be forced into the hook by

main strength.

Replacing zippers &&

-----------------

General

-------

If you replace a fastener more than once, consider

replacing it with a different kind of fastener.

Bed linen

=========

It is rarely a good idea to mend a worn sheet.  Sheets

wear out so uniformly that when a hole appears in one part,

the rest is about to go.  Accidental injuries are worth

repairing, however, particularly when they are near the edge

and you won't have to lie on the patch or darn.

If you are short of sheets and must repair one that

has worn thin in the middle, tear it lengthwise, sew

the selvages together, and hem the torn edges.  This

puts the strong fabric where the strain is, and leaves

the thin fabric to tuck under the mattress.  It also

puts a seam right where you lie -- that's another thing

wrong with mending sheets:  they wear through exactly

where a patch or darn is most uncomfortable.  Use an

overcast seam or lap seam to minimize the discomfort.

The good parts of worn sheets can be made into

pillowcases, but the pillowcases generally wear out very

soon, so don't invest much labor in them.

Bandages are a traditional use for worn sheets:  they

are torn lengthwise to make roller bandages, or they are

torn into forty-inch squares and the squares are cut on the

diagonal to make triangular bandages.  Sometimes triangular

bandages are called "cravat" bandages, but strictly

speaking, they are cravats only when folded into a narrow

strip parallel to the long side.

Before making bandages, the old sheet should be

washed with hot water and bleach, and thoroughly

rinsed.  (You may wish to soak in bleach or other

disinfectant cold, then use the hottest-available water

for the first rinse.)  Baste around any holes first to

keep them from growing.  For extra credit, iron the

freshly-made bandages; this comes pretty close to

sterilizing them.  Store bandages in new food-grade

plastic bags.

If there is no immediate use for a worn-out sheet, store

it on a top shelf; sooner or later you are going to want a

very large rag.  We used to use old sheets to simulate snow

under Christmas trees, for example -- this led to the

"Christmas-tree skirt".  Old sheets are also traditional

dust covers for disused furniture.  If you make tailored

clothing from dry-clean-only fabric, you'll need an old

all-cotton sheet to "sponge" the fabric.

Worn-out all-cotton sheets and pillowcases make

excellent "blow" handkerchiefs, and a piece of sheet in the

rag drawer is a convenient source of impromptu bandages and

semi-disposable cleaning rags.

A *linen* sheet, of course, is worth more trouble.

Family legend has it that one of my ancestors made a

set of dinner napkins from the good parts of a linen

sheet that her ancestor had brought with her from "the

old country" -- possibly Ireland.

----------------------------------------------------------

Mending Woven Wool

==================

I use DMC's "Medici" embroidery wool to sew patches on my

wool house pants.

Medici is a hard-twisted yarn about the thickness of carpet

thread.  When I use black Medici on fuzzy black wool (flannel

washed in hot water), it is hard to see, and it tends to felt

into the fabric, so I needn't be fanatical about securing

the ends.

For my rough-and-ready mending, I cut a patch larger than

the worn area, lay it over the hole, back-stitch it to the

garment, then overcast all raw edges down.  Since wool

doesn't ravel much, this would work even if the fabric

hadn't been felted.

An accidental injury can be mended by cutting a patch to the

exact size of the hole.   (Make *very* sure that the patch

isn't going to change size when the garment is washed!)

More precisely, cut it ever so slightly bigger than the

hole, so that you have to squish it in, but it still will

lie perfectly flat without coercion.

Set the patch into the hole, raw edges just meeting, and

darn it in by slipping your needle through the fabric, so

that the thread doesn't show on either side, from one side

of the cut to the other.  Put the needle down very close to

where it came up and slide through the fabric back to the

other side of the slit.  (One name for this is "rantering

stitch".  Some think it's short for "re-entering stitch".)

Work from the wrong side so that the little bumps of thread

where you turn will be inside the garment.

For darning-in, buttonhole silk is much stronger than

Medici, which you can easily break with your bare hands.

(And you should, since a fuzzy broken end is easier to hide

than an end cut square.)  And silk thread is easier to slide

through the fabric.  Don't use nylon or polyester thread.

I read somewhere that a human hair was the best thread for

invisible mending.  I had a couple of eighteen-inch white

hairs once, and tried sewing with them in lieu of the size A

white silk that was short supply.  I found it almost

impossible to thread a needle with two hairs -- they wanted

to go in different directions -- and yes, the "thread" is

invisible, as in impossible to see what you are doing.  I

imagine a single hair would be easier to handle, and worth

the effort if repairing a fine antique.  (#100+ silk is

almost as hard to sew with as human hair -- but more

conveniently packaged.)

If rantering won't work, baseball stitch is another way to

darn a slit.  The thread will fill a slight gap; a larger

gap can be filled by weaving over and under a vertical

thread.  I prefer wool for baseball stitch, since it shows

less than silk, it's easier to secure, and strength isn't as

important as in rantering.

I foolishly wore a white challis headscarf while burning

brush, and a spark burned a small hole in it.  I darned the

hole -- parallel rows of running stitch intersecting in a

woven darn such as you would use for linen -- with white

size A sewing silk, and much to my surprise, I have to look

very closely to see that the scarf has been mended.

I don't like sharp angles in my sewing -- I leave holes

rounded, and trim out only what's hopeless or lumpy.

However, all the old books suggest trimming to a rectangle;

could be the authors knew something.

To prevent puckers and tearing-away after washing,

choose a patch that has been washed as many times as the

garment, match the grain, and hope for the best.

> I've tried sewing with a thread pulled from the garment

> itself, but OH is that frustrating.  Breaks, a lot...

Wax helps -- slicks down the fuzz, and gluing the fibers

together inhibits fraying.  Using short pieces also helps,

as do using double threads and choosing threads that run

parallel to the selvage.  I never tried it with wool,

however.

I just dug out a thin wool blanket I bought at a garage

sale.  It was probably mended some time in the thirties, but

it might have belonged to a farmer, in which case hard times

went a *lot* farther back.  On the other hand, it's

definitely machine stitching.

Whoever mended it cut two corners off the blanket,

overlapped the bias edges half an inch, and sewed close to

each edge -- too close on one side; parts of it have come

unstitched.

Then he or she trimmed the hole to a square, laid the

square made from the corners over it (overlapping by an inch

and a half), and sewed twice around the edges of the patch

and the hole.  One row of stitching was a quarter inch from

the edge, and one row was right on the edge.

He finished by single-crocheting -- looks like blanket

stitch at first glance -- completely around the entire edge

of the blanket.  Wish I knew how he worked so *evenly*.  He

made a chain stitch after each single crochet, which would

have made the work a bit faster, and the edge is nearly half

an inch wide.

It has held up quite well; in fact, I think I'll skip

re-sewing the stitches that have let go, since the wool

isn't going to fray, and the remaining stitches are holding

quite well.

Besides, the blanket is wearing into small holes.

------------------------------------

EOF